Calculating or Determining the 90th Percentile

during Initial, Follow-up, Routine, and

Reduced Monitoring

If you collect 5 samples, calculate your 90th percentile like this:

- Rank your samples in order of concentration (mg/L), starting from lowest to highest.

- Find the average of the two highest results by adding them together and dividing by 2.

- The resultant number is recorded for the 90th percentile.

Sample Site #		Sample results

| 1 | 0.001 |
| --- | --- |
| 2 | 0.001 |
| 3 | 0.006 |
| 4 | 0.008 |
| 5 | 0.014 |

|  | 0.008 |  |
| --- | --- | --- |
| + This is the number to record on Form 141A. | 0.014 |  |
|  | 0.022 |  |
| Average | divide 0.022 by 2 = 0.011 |  |

(This is an example.  Insert your own sample results.)

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * ** * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

Calculating the 90th Percentile for 6 samples or more:

- Rank your samples in order of concentration starting from lowest to highest.

- Take the total number of samples collected and multiply it by 0.90.  The resultant number will tell you which sample to record.

- If the number is not a whole number, you need to round to the nearest whole number.

- If the number is exactly in the middle of two whole numbers, you round to the nearest even number.

- i.e.  12.5 would be rounded to 12, and 13.5 would be rounded to 14.

Example:  If you collect 10 samples, determine your 90th percentile like this:

- 10 x 0.90 = 9

Sample Site #		Sample results

| 1 | 0.001 |
| --- | --- |
| 2 | 0.001 |
| 3 | 0.001 |
| 4 | 0.001 |
| 5 This sample should be recorded as the 90th percentile result on Form 141A. | 0.001 |
| 6 | 0.004 |
| 7 | 0.005 |
| 8 | 0.006 |
| 9 | 0.008 |
| 10 | 0.010 |

(This is an example.  Insert your own sample results.)

(Use this form if your laboratory does not provide a 90th percentile summary for you)

Public Water Supply ID No. __________________

Name of Public Water Supply: ____________________________________________________________

Results of Monitoring:

|  | Date Collected | Sample Location | Lead Result | Which Tier? |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

90th Percentile for Lead:

|  | Date Collected | Sample Location | Copper Result | Which Tier? |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

90th Percentile for Copper:

Mail to :	Water Supply Operations Section

Iowa Department of Natural Resources

502 E. 9th St.

Des Moines, IA 50319-0034

EPA/DNR form 141-A, p.3

PWSID # _______________

NAME OF PUBLIC WATER SUPPLY: ______________________

Month & Year Samples were Collected:  ___________________

RESULTS OF MONITORING

# Samples Required _______# Samples Submitted _____		90th Percentile Lead _______ mg/L

90th Percentile Copper ______ mg/L

CHANGE OF SAMPLING SITES

Original Site Address:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

New Site Address:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Distance between Sites (approximately):

____________________________________________________________________________________

Targeting Criteria: 	NEW: ______________________________________

OLD:  ______________________________________

Reason for Change (attach additional pages if necessary): ________________________________________________________________________________________________________________________________________________________________________

SAMPLES SITE IDENTIFICATION AND CERTIFICATION

SIGNATURE (name & title )					Today’s Date: ________________________

____________________________________________________________________________________

DNR Field Office_______